USER MANUAL

Intelligent driving system for all-domain intelligent logistics and autonomous driving

Team Intelligent Mobility in All Domains

CPWC2025-0001

Index

Introduction2

Overview and Objectives of the Project 2

System Requirement2

Hardware Requirements2

Software Requirements3

Installation Guide 3

UC-win/Road COM Installation 3

How to use the LLM Model 4

Vehicle Data Collection 4

Code Explain4

Main Classes and Functions19

LLM Fine-Tuning 20

Explanation of the Fine-Tuning Method20

Dataset Preparation20

GUI Integration and User Guide 21

Explanation of GUI Structure21

Description of GUI Functions and Examples 21

Reference 24s

1. Introduction

1.1 Overview and Objectives of the Project

This project proposes a truck autonomous driving decision system based on a Vision-Language Model (VLM), with Mini-InternVL2-DA-BDD (DriveLM direction) as the core model, validated in the high-fidelity UC-win/Road simulation platform.

The system adopts a three-layer architecture:

Data Layer: Collects vehicle state, road structure, and traffic participants from UC-win/Road, then converts them into structured features and natural language prompts.

Reasoning Layer: Uses a fine-tuned VLM to transform multi-modal context into natural language driving suggestions.

Execution Layer: Parses the suggestions into vehicle control signals (throttle, brake, steering) and applies them to the simulated vehicle.

Objectives:

Improve intelligent decision-making and safety in complex logistics scenarios.

Achieve efficient inference under constrained compute (RTX 4060 8GB) while maintaining near large-model performance.

Support unmanned upgrades across ports, industrial parks, and highway–urban distribution logistics.

2. System Requirement

2.1 Hardware Requirements

Recommended:

GPU: NVIDIA RTX 4060 (8 GB VRAM) or higher

CPU: 8 cores or more

RAM: ≥16 GB

Storage: SSD ≥50 GB free space (for model and data)

Simulation and sensors:

UC-win/Road typically runs on Windows (see installation).

2.2 Software Requirements

Operating Systems:

Inference/Training: Python 3.9, CUDA 12.x

Simulation: UC-win/Road on Windows 10/11 (with COM/Python interface license)

Python dependencies (typical):

torch==2.2.2 (matching your CUDA)

transformers, accelerate, peft, bitsandbytes

pillow, opencv-python, pandas

Windows-only: pywin32 or comtypes for COM/Automation

UC-win/Road:

Version with COM/Python API support and valid license (recommend v16+)

3. Installation Guide

3.1 UC-win/Road COM Installation

Steps (on Windows):

Install UC-win/Road with COM/Python API support and license.

Ensure the COM interface is registered. If not, register via the official scripts (run as Administrator). Refer to official SDK docs for exact commands.

Prepare a Python environment (conda/venv recommended) and install Windows COM bindings and common libs:

bash

pip install pywin32 comtypes pillow opencv-python pandas

Open a UC-win/Road sample project and verify a simple COM call using official examples or your integration scripts.

Note:

Actual COM object model and method names should follow the UC-win/Road SDK. This manual references your custom wrapper functions (e.g., GetVehicleInfomation, GetRoadAroundVehicle) mapped in your own Python layer.

3.2 How to use the LLM Model

Environment setup (Windows or Linux):

bash

# Select a torch build matching your CUDA

pip install torch==2.2.2 --index-url https://download.pytorch.org/whl/cu121

pip install transformers accelerate peft bitsandbytes

Place your Mini-InternVL2-1B-DA-DriveLM model files locally and set path in your script to that directory.

The model is loaded with torch.bfloat16 and use_flash_attn=True in your code. Ensure:

A compatible GPU is available and drivers/CUDA are correctly installed.

FlashAttention support is available for your environment, or set use_flash_attn=False if not.

4. Vehicle Data Collection

4.1 Code Explain

The following is your complete integration code for UC-win/Road data collection, prompt construction, VLM-based reasoning, and vehicle control. It:

Loads the Mini-InternVL2-1B-DA-DriveLM from a local path.

Gathers ego vehicle and nearby traffic objects via UC-win/Road COM.

Builds a concise English prompt describing state and environment.

Calls model.chat to obtain a driving suggestion.

Parses the suggestion into discrete actions and applies control to the vehicle.

python

from UCwinRoadCOM import *

from LoggerProxy import LoggerProxy

from VehicleInfo import DataclassVehicleInfo

import torch

import time

import datetime

import pandas as pd

import math

from transformers import pipeline

from transformers import AutoModel, AutoTokenizer

# Define the model path

path = 'E:\\UCwin\\Plugins\\PythonAPIPlugin\\Mini-InternVL2-1B-DA-DriveLM'

# Loading the model and tokenizer

model = AutoModel.from_pretrained(

path,

torch_dtype=torch.bfloat16,

low_cpu_mem_usage=True,

use_flash_attn=True,

trust_remote_code=True

).eval().cuda()

tokenizer = AutoTokenizer.from_pretrained(path, trust_remote_code=True, use_fast=False)

def Distance(pos1, pos2):

return math.sqrt((pos2[0] - pos1[0]) ** 2 + (pos2[1] - pos1[1]) ** 2 + (pos2[2] - pos1[2]) ** 2)

def StrCarLights(carLights):

state : str  =  ''

if carLights.isLeftIndicatorOn:

state = 'LeftIndicator/'

if carLights.isRightIndicatorOn:

state = state + 'RightIndicator/'

if carLights.isBrakeLightOn:

state = state + 'BrakeLight/'

if carLights.isHighBeamOn:

state = state + 'HighBeam/'

if carLights.isLowBeamOn:

state = state + 'LowBeam'

return state

def GetNextRoadPoint(vehicle, range_search=50):

""" Get the road point information in front of the vehicle """

prj = winRoadProxy.Project

next_point = None

# Get the current road

current_road = vehicle.currentRoad

if not current_road:  # If the current segment does not exist

logProxy.logger.warning("Vehicle is not on any road.")

return next_point

roads = prj.RoadsCount

for i in range(roads):

road = prj.Road(i)

if road.Name != current_road:

continue

road_length = road.Length

# Traverse the road and find the point in front of the vehicle

for dist in range(0, int(road_length), 5):  # Sampling every 5 meters

road_pos = road.GetPositionAt(dist)

if Distance((road_pos.X, road_pos.Y, road_pos.Z), (vehicle.Pos_X, vehicle.Pos_Y, vehicle.Pos_Z)) < range_search:

next_point = (road_pos.X, road_pos.Y, road_pos.Z)

break

if next_point:

break

return next_point

def GetRoadAroundVehicle(vehicle, range_search=50):

"""

Get information about roads with the same name within the range around the vehicle

:param vehicle: Car information

:param range_search: Range (m)

:return: Road Information List

"""

prj = winRoadProxy.Project

road_info = []  # List used to store road information

# Get the current road

current_road = vehicle.currentRoad

if not current_road:  # If the current segment does not exist

logProxy.logger.warning("Vehicle is not on any road.")

return road_info

current_road_name = current_road

logProxy.logger.info(f"Current road: {current_road_name}")

# Iterate over all roads and find the road with the same name as the current road

roads = prj.RoadsCount

for i in range(roads):

road = prj.Road(i)

# Only process roads whose names match the current road

if road.Name != current_road_name:

continue

roadLength = road.Length

dist = 0.0

logProxy.logger.info(f"Checking road: {road.Name}, Length: {roadLength}")

# Only traverse to a certain distance to avoid unnecessary calculations

while dist <= roadLength:  # Make sure not to exceed the road length and search radius plus 50 meters

roadPos = road.GetPositionAt(dist)

roadPos_position = (roadPos.X, roadPos.Y, roadPos.Z)

vehicle_position = (vehicle.Pos_X, vehicle.Pos_Y, vehicle.Pos_Z)

# Calculate the distance between the vehicle and the road point

if Distance(roadPos_position, vehicle_position) <= range_search:

angle = math.atan2(road.GetDirectionAt(dist).X, -road.GetDirectionAt(dist).Z) * 180 / math.pi

if angle < 0:

angle += 360

curvature = road.GetCurvatureAt(dist)

strRadius = 'INF' if curvature == 0 else str(1 / curvature)

slope = road.GetSlopeAt(dist) * 100

road_info.append({

'Name': road.Name,

'Position': (roadPos.X, roadPos.Y, roadPos.Z),

'Curvature': strRadius,

'Slope': slope,

'Angle': angle

})

logProxy.logger.info(f"Added road info: {road_info[-1]}")  # Logs added road information

dist += 50  # Sampling every 10 meters to reduce the amount of calculation

return road_info  # Sampling every 10 meters to reduce the amount of calculation

def GetVehicleInfomation():

# Setting Pandas Printing Options

pd.set_option('display.max_rows', None)

# Set the maximum number of lines to None, that is, print all lines    pd.set_option('display.max_columns', None)

# Set the maximum number of columns to None, that is, print all columns

pd.set_option('display.width', None)  # Adaptive column width

pd.set_option('display.max_colwidth', None)  # Allow column widths to not be truncated

const = winRoadProxy.const

mainCam = winRoadProxy.MainForm.MainCamera

cameraSts = mainCam.MainCameraState

eyework = cameraSts.eye

traffic = winRoadProxy.SimulationCore.TrafficSimulation

trafficList = traffic.GetTransientObjectsArround(50, eyework)  # 50 meters nearby

count = trafficList.Count

logProxy.logger.info("count={}".format(count))

user_vehicle = None  # User vehicles

nearby_vehicles = []  # Other vehicles

if count > 0:

viList = []

for i in range(count):

obj = trafficList.Items(i)

#logProxy.logger.info(obj)

if obj is None:

continue

transientType = obj.TransientType

if const._TransientCar == transientType :

vi = DataclassVehicleInfo();

vi.Timestamp = datetime.datetime.now()

vi.ID = obj.ID

vi.Description = obj.Description

pos = obj.Position

vi.Pos_X = pos.X

vi.Pos_Y = pos.Y

vi.Pos_Z = pos.Z

vi.Yaw = obj.YawAngle

vi.Pitch = obj.PitchAngle

vi.Roll = obj.RollAngle

direction =  obj.Direction

vi.Dir_X = direction.X

vi.Dir_Y = direction.Y

vi.Dir_Z = direction.Z

vi.BodyPitch = obj.BodyPitchAngle

vi.BodyRoll = obj.BodyRollAngle

vi.EngineRPM = obj.RPM

speed = obj.SpeedVector(const._MeterPerSecond)

vi.Speed_X = speed.X

vi.Speed_Y = speed.Y

vi.Speed_Z = speed.Z

accel = obj.Acceleration

vi.Acceleration_X = accel.X

vi.Acceleration_Y = accel.Y

vi.Acceleration_Z = accel.Z

bRotSpeed = obj.BodyRotSpeed

vi.BodyRotSpeed_X = bRotSpeed.X

vi.BodyRotSpeed_Y = bRotSpeed.Y

vi.BodyRotSpeed_Z = bRotSpeed.Z

bRotAccel = obj.BodyRotAcceleration

vi.BodyRotAccel_X = bRotAccel.X

vi.BodyRotAccel_Y = bRotAccel.Y

vi.BodyRotAccel_Z = bRotAccel.Z

rotSpeed = obj.RotSpeed

vi.RotSpeed_X = rotSpeed.X

vi.RotSpeed_Y = rotSpeed.Y

vi.RotSpeed_Z = rotSpeed.Z

rotAccel = obj.RotAcceleration

vi.RotAccel_X = rotAccel.X

vi.RotAccel_Y = rotAccel.Y

vi.RotAccel_Z = rotAccel.Z

vi.DistTravelled = obj.DistanceTravelled

vi.Steering = obj.Steering

vi.Applied_steering = obj.AppliedSteering

vi.Throttle = obj.Throttle

vi.AppliedThrottle = obj.AppliedThrottle

vi.Brake = obj.Brake

vi.AppliedBrake = obj.AppliedBrake

vi.Clutch = obj.Clutch

vi.AppliedClutch = obj.AppliedClutch

carLights = obj.CarLights

vi.LightState = StrCarLights(carLights)

intersection = obj.CurrentIntersection;

if intersection is not None:

vi.Intersection = intersection.Name

vi.AutomaticControl = 'Automatic' if obj.AutomaticControl else 'Manual'

vi.Mass = obj.Mass

curRoad = obj.CurrentRoad

if curRoad is not None:

vi.currentRoad = curRoad.Name

vi.DistAlongCurrentRoad = obj.DistanceAlongRoad

lastRoad = obj.LatestRoad

if lastRoad is not None:

vi.LatestRoad = lastRoad.Name

vi.DistAlongLatestRoad = obj.DistanceAlongLatestRoad

vi.LaneNumber = obj.LaneNumber

vi.LaneWidth = obj.LaneWidth

laneDir = obj.LaneDirection

vi.LaneDir_X = laneDir.X

vi.LaneDir_Y = laneDir.Y

vi.LaneDir_Z = laneDir.Z

vi.LaneCurvature = obj.LaneCurvature

bodyDir = obj.BodyDirection

vi.BodyDir_X = bodyDir.X

vi.BodyDir_Y = bodyDir.Y

vi.BodyDir_Z = bodyDir.Z

vi.DistanceAlongPath = obj.DistanceAlongDrivePath

# curLane = obj.CurrentLane

# if curLane is not None:

#     if curLane.CurveType == const._cfRoad:

#         vi.LaneID = curLane.GetLaneID(vi.DistanceAlongPath, const._ldLane)

# vi.EngineOn = obj.EngineOn

vi.SpeedInKmph = obj.Speed(const._KiloMeterPerHour)

if "[user's car]" in vi.Description:

user_vehicle = vi  # Identify the user's vehicle

vehicle_controller = obj  # Get the vehicle control proxy object

else:

nearby_vehicles.append(vi)  # Record nearby cars

# viList.append(vi)

del vi

if user_vehicle:

# Build Tips

prompt = f"""

I am driving a {user_vehicle.Description} with ID {user_vehicle.ID} at a speed of {user_vehicle.SpeedInKmph:.2f} km/h.

My position is at coordinates ({user_vehicle.Pos_X:.2f}, {user_vehicle.Pos_Y:.2f}, {user_vehicle.Pos_Z:.2f}).

My current road is {user_vehicle.currentRoad}.

The yaw angle is {user_vehicle.Yaw:.2f} degrees. Other nearby vehicles include:

"""

# Add nearby vehicle information to reminders

for v in nearby_vehicles:

prompt += f"""

- {v.Description}: Speed {v.SpeedInKmph:.2f} km/h at coordinates ({v.Pos_X:.2f}, {v.Pos_Y:.2f}, {v.Pos_Z:.2f}).

"""

# Add road information to prompts

# Current road information

# Get road information before processing vehicles

road_info = GetRoadAroundVehicle(user_vehicle)

print(road_info)

prompt += "Currently nearby roads:\n"

for road in road_info:

prompt += f"""

- Road Name: {road['Name']}, Position: ({road['Position'][0]:.2f}, {road['Position'][1]:.2f}, {road['Position'][2]:.2f}),

Curvature: {road['Curvature']}, Slope: {road['Slope']} degrees, Direction Angle: {road['Angle']} degrees.

"""

# prompt += """

# Based on this information, please provide one actions I should take to drive safely and effectively.

# """

prompt += """

Based on the information provided, please respond with simple and direct commands using the following actions:

- accelerate

- decelerate

- keep speed

- turn left

- turn right

- shift left

- shift right

"""

response = model.chat(tokenizer, None, prompt, dict(max_new_tokens=1024, do_sample=True))

# Output the model's answer            print(f'User: {prompt}\nAssistant: {response}')

# Control the vehicle according to the model's recommendations

control_vehicle(response, vehicle_controller)

def parse_driving_instructions(response):

""" Parse the driving suggestions provided by LLM and return a list of instructions """

instructions = []

action_phrases = {

"accelerate": ["accelerate", "increase speed"],

"decelerate": ["decelerate", "slow down"],

"keep speed": ["keep going", "same speed"],

"turn left": ["turn left"],

"turn right": ["turn right"],

"shift left": ["shift left", "slightly offset to the left"],

"shift right": ["shift right", "slightly offset to the right"],

}

# Examine each phrase and decide what action to take

for action, phrases in action_phrases.items():

for phrase in phrases:

if phrase in response:

instructions.append(action)

return instructions

def control_vehicle(response, vehicle_controller):

""" Adjust the state of the ego vehicle based on the model's response """

# Convert the command in the response to lowercase and remove extra spaces    action = response.strip().lower()

logProxy.logger.info(f"Control action received: {action}")

# Parsing instructions

instructions = parse_driving_instructions(action)

# Execution control logic

for instruction in instructions:

if instruction == "accelerate":

logProxy.logger.info("Increasing speed.")

current_speed_vector = vehicle_controller.SpeedVector(const._MeterPerSecond)

current_speed = current_speed_vector.X

vehicle_controller.SetSpeed(const._MeterPerSecond, current_speed + 10.0)

elif instruction == "decelerate":

logProxy.logger.info("Decreasing speed.")

current_speed_vector = vehicle_controller.SpeedVector(const._MeterPerSecond)

current_speed = current_speed_vector.X

vehicle_controller.SetSpeed(const._MeterPerSecond, max(0, current_speed - 10.0))

elif instruction == "keep_speed":

logProxy.logger.info("Maintaining current speed.")

# Make no changes

elif instruction == "turn left":

logProxy.logger.info("Turning left.")

vehicle_controller.Steering -= 25  # Adjust 15 degrees each time

elif instruction == "turn right":

logProxy.logger.info("Turning right.")

vehicle_controller.Steering += 25  #  Adjust 15 degrees each time

elif instruction == "shift left":

logProxy.logger.info("Shifting slightly to the left.")

# Implementing slight offset logic

vehicle_controller.Steering -= 10

elif instruction == "shift right":

logProxy.logger.info("Shifting slightly to the right.")

# Implementing slight offset logic

vehicle_controller.Steering += 10

if not instructions:

logProxy.logger.info(f"Received unrecognized action: {action}")

def main():

try:

start = time.perf_counter_ns()

global winRoadProxy

winRoadProxy = UCwinRoadComProxy()

global const

const = winRoadProxy.const

scriptName = 'Sample_OutputVehicleInfo'

global logProxy

logfilepath = winRoadProxy.PythonPluginDirectory() + scriptName + '.log'

logProxy = LoggerProxy(scriptName, logfilepath)

logProxy.logger.info('Start '+ scriptName)

GetVehicleInfomation()

elapsed_time = time.perf_counter_ns() - start

logProxy.logger.info("Total:{}ms".format(elapsed_time/1000000))

logProxy.logger.info('End '+ scriptName)

finally:

logProxy.killLogger()

del winRoadProxy

if __name__ == '__main__':

main()

4.2 Main Classes and Functions

GetVehicleInfomation():

Enumerates transient objects within 50 m of the main camera’s eye point.

Extracts detailed ego and nearby vehicle states into DataclassVehicleInfo.

Builds an English prompt with ego pose, speed, road, plus nearby vehicle list and road geometry from GetRoadAroundVehicle.

Calls model.chat(...) and passes the response to control_vehicle(...).

GetRoadAroundVehicle(vehicle, range_search=50):

Filters roads with the same name as the vehicle’s current road.

Samples along the road and collects position, curvature (radius), slope, and direction angle if within range.

GetNextRoadPoint(vehicle, range_search=50):

Finds the next road point ahead on the current road within a search radius.

parse_driving_instructions(response):

Parses LLM’s text response into a list of discrete actions: accelerate, decelerate, keep speed, turn left/right, shift left/right.

control_vehicle(response, vehicle_controller):

Applies parsed actions to the UC-win/Road vehicle controller:

Speed adjustments via SetSpeed(...)

Steering changes for turning and lateral shifts

Utility:

Distance, StrCarLights for helper computations and light-state formatting.

Entry point:

main() initializes UCwinRoadComProxy, logging, calls GetVehicleInfomation(), and cleans up.

5. LLM Fine-Tuning

5.1 Explanation of the Fine-Tuning Method

Strategy: Instruction-tune Mini-InternVL2-DA-BDD so that the model maps scene description + state to concise, safe driving commands.

Resource constraints: On a single 8 GB GPU, consider parameter-efficient fine-tuning (e.g., LoRA) and quantization-aware training to reduce memory usage.

Objective: Stable, safe, and interpretable suggestions in natural language that map cleanly to the discrete action set used in your controller.

5.2 Dataset Preparation

Data sources:

UC-win/Road screenshots (first- and third-person views), synchronized with vehicle states.

DriveGPT4-style generated or expert-annotated action suggestions to ensure high-quality targets.

Data design:

Each sample pairs a structured scene description (or multi-modal inputs) with a short action response limited to your action set to simplify parsing.

Cover diverse conditions: straight/curves, traffic lights, following/merging, pedestrian crossings, ramps, adverse weather, and nighttime.

Best practices:

Enforce response templates that align with your parse_driving_instructions lexicon.

Human-in-the-loop curation to remove ambiguous or multi-action outputs unless your controller supports sequencing.

6. GUI Integration and User Guide

6.1 Explanation of GUI Structure

Recommended technologies: PyQt5/Qt for Python or a lightweight web GUI (e.g., Gradio) for rapid iteration.

Typical layout:

Left: Live simulation view (first-/third-person).

Right top: Status panel (speed, pose, lane, lights, nearby entities).

Right bottom: LLM suggestion and control log.

Toolbar: Connect/Disconnect, Start/Stop, Manual Override, Record, Load Scenario, Emergency Brake.

Bottom: Model selector (base + LoRA), parameters, log export.

6.2 Description of GUI Functions and Examples

Core functions:

Connect to UC-win/Road COM instance.

Start/Stop inference loop synchronized to simulation steps.

Display real-time vehicle/environment status and model suggestions.

Show control actions applied; support manual override/emergency brake.

Record and export logs (CSV) and frames (for replay and evaluation).

Usage flow:

Launch UC-win/Road and load your scenario.

Start the GUI and connect to the running simulation instance.

Choose the local model path and (optional) fine-tuned weights.

Start inference; observe suggestions and vehicle behavior.

Use emergency/manual controls if needed; export run logs afterward.

Fig1. UC-win/Road usage diagram

Fig2. Schematic diagram of the train station scene

- Self-driving truck first-person perspective

- Self-driving truck third-person perspective

Fig3. Rendering of autonomous truck driving

7. Reference

- State Council of the PRC: “14th Five-Year Plan for Modern Logistics Development”https://www.gov.cn/zhengce/content/2022-12/15/content_5732092.htm

- Xu Z, Zhang Y, Xie E, et al. DriveGPT4: Interpretable end-to-end autonomous driving via large language model. IEEE Robotics and Automation Letters, 2024.

- OpenGVLab/Mini-InternVL2-1B-DA-DriveLM · Hugging Face.

- InternVL overview https://zhuanlan.zhihu.com/p/706547971

- InternVL GitHub https://github.com/OpenGVLab/InternVL

- Chen Z, Wang W, Tian H, et al. How far are we to GPT-4V? Science China Information Sciences, 2024, 67(12): 220101.

Notes and Disclaimer

This system is for research and demonstration. Real-world deployment must comply with local laws and integrate redundant safety mechanisms.

UC-win/Road COM/Python API names must follow the official SDK. The functions shown here are part of your integration and should be mapped accordingly.

Ensure your action vocabulary in fine-tuning matches the parser in parse_driving_instructions to guarantee reliable control.