Precision-Driven Digital Twin System for Autonomous Vehicle Validation: Synthesizing High-Fidelity Geospatial and UAV-Derived Data——User manual

Program Application Introduction：

- Static scene construction: In order to build high-precision simulation scenes, we utilize the block map and 3D model import function of UC-win/Road, combined with the road simulation function of UC-win/Road, to quickly construct high-precision terrain data and configuration of road attachments.

- Dynamic traffic flow construction: In order to construct high-precision simulated traffic flow, we use publicly available drone aerial traffic flow data, combined with the event simulation function of UC-win/Road, and assisted by HXd editor to edit event RSO files. We use Python and LLM to analyze the features of the aerial data and generate RSO files in batches, and model the vehicle's driving trajectory, speed distribution, and interaction behavior to achieve dynamic restoration of real traffic flow.HMI Project: In order to achieve human-computer interaction under autonomous driving conditions, VisualTFT.exe is used to design the interfaces in automatic driving mode and manual driving mode, based on LUA scripting is written for obtaining CAN bus data and UDP data, compiling the project file and downloading it to a U Disk, and uploading it to the instrument panel using the U Disk.

- QT Controller: In order to reduce the pressure of data transmission and calculation of the UC custom plug-in, the related program written in Qt is used, in which Test.exe receives the planning path from UC via UDP to realize vehicle following and transmits the control information to the driving simulator via TCP, and the UDP is transmitted back to the UC plug-in, and Qt_Client.exe transmits the data transmitted from UC to the HMI interface via UDP. Real-time display.

- UC Plugin: UC custom plug-ins through UDP and Qt control plug-ins to achieve data transfer; through the driver's manual driving data to determine the driving style, and according to different driving styles to develop different decision-making path.

## Static scene construction

1. Open the application and complete the road modeling based on the latitude and longitude of the real road. The latitude, longitude, and flat curve elements of the corresponding points can be constructed using open-source data such as open street maps or Google Maps.

2. After completing the road modeling, you can choose to download 3D models such as 3ds/fbx/BIM from the library, or import them externally into UC-win/Road. In order to demonstrate high-precision scene modeling, this project chooses to import the corresponding real road attachments from the outside. Then, by using the road accessory function to quickly set its distribution road, vertical distance, horizontal distance and other elements, road accessories can be generated quickly.

3. Afterwards, we use the block map function of UC-win/Road to import images of selected areas to replace the original terrain model. We then select the block map in the editing interface, build block areas of image size, and select images for the area

## Dynamic traffic flow import：

1. Choose to download the public aerial photography dataset, and after downloading, analyze the data and split it into the content we need. This also needs to be completed in conjunction with the second point of event RSO file parsing.

2. Use the Hxd editor to parse the event RSO file. The scene event function of UC-win/Road not only supports manual editing, but also supports outputting and reading RSO files. The RSO file here is generated through binary editing and cannot be simply read through window text. It can be easily replaced and written using the Hxd editor. We analyzed the characteristics and composition of the RSO file of UC-win/Road using Hxd, corresponded the front-end editing content to the RSO file content, and combined it with aerial data to batch modify the RSO file.

- However, through HXd file by file modifications, for massive aerial data, we need to use other tools for batch processing. Do we generate RSO files in batches using Python and LLM models? We connect to Python through LLM's API interface, call Deepseek's large model to analyze and batch process aerial raw data, and then read and write the text content for RSO based on Python. Finally, let UC-win/Road read the RSO file to generate high-precision background traffic flow.

UC Plugin

- Open Program application/UC Plugin/UC Plugin Application/ UC_devlop_16.01 → Generate a project file after writing program.

- Copy the Program application/UC Plugin/Executable Document to the UC source directory.

- Open UC-win/Road → Open Program application/UC Scenario/Two- way four-lane road.rd/ Two- way eight-lane road.rd → Click to trigger scenario event (If it is Two- way eight-lane road.rd, remember to generate virtual traffic flow)→ Click “C++APL_UDP_contact” to open the custom plugin interface. → Click “UDP Bind” connecting with QT plug-ins. → Click “ACC	_Drive” receiving control information. → Select “Aggressive/Normal/Conservation” Driving Style.

HMI Project

The specific process as following.

- Open Program application/HMI → Project.Setup VisualTFT then open “VisualTFT.exe”.

- Open “xxxx.tft” in “Screen Files” to design the different mode screen. Using the tools to insert different text, pictures or icons.

- Open "Tools-LUS Script" to write control scripts, please refer to the help file in "Help" for the specific API.

- Click "Load" to compile the project file, select "U Disk or SD card download", copy "DCIOT.PKG" to U Disk.

- Upload the PKG file on the HMI motherboard and the interactive information can be displayed after the upload is completed.

Auto-mode 							Manual-mode

QT Controller

- Open Program application/QT Project/ QT Controller to develop control algorithms → Generating Executable Document.

- Open Program application/QT Project/ QT Client to Write messages to be passed to the HMI. → Generating Executable Document.

- Open Program application/QT Project/ Executable Document/QT Controller. → Click “One-touch opening”(一键式开启) Start receiving data from UC to calculate and transmit control information.

- Open Program application/QT Project/ Executable Document/QT Client. → Click “UDP Start”(UDP打开) Start receiving data from the UC and calculate and transmit display information to the HMI.

Autonomous driving and takeover

Before the experiment the driver selects the initial driving style, in the driving simulation cabin, presses the autopilot button to enter the autopilot mode, and adjusts the following distance and cruising speed on the steering wheel. When dissatisfied with the system control turn the steering wheel or step on the brake to take over, the system will adjust the decision planning parameters by manual driving data, the switching buttons are as follows.

Auto-mode switching buttons