USER MANUAL

Speed Bump Response System for Autonomous Driving

Team AutoBump

CPWC2025-0039

Index

- Introduction

  - Overview

  - Objectives of the Project

- System Requirement

  - Hardware Requirements

  - Software Requirements

- Installation Guide

- System Architecture

  - Module Roles and Key Functions

  - Key Variables List

- Code Details

Introduction

Overview

This document is the user manual for the " Speed Bump Response System for Autonomous Driving." This system aims for an autonomous vehicle to recognize and respond to various types of speed bumps through information from sensor data (measured) and data provided from the simulator (Ground Truth), without prior information such as HD Maps.

This is a comprehensive 'Response' system that not only secures ride comfort through smooth deceleration but also considers driving efficiency by minimizing unnecessary braking. The core of the system is a Self-learning Loop consisting of ‘Perception -> Classification (Type of speedbump) -> Control -> Evaluation -> Calibration’. Through this process, the system adapts to changing road environments and optimizes its performance as driving experience accumulates.

Objectives of the Project

Conventional speed bump response systems in vehicles utilize both sensor information and HD maps, but this approach has limitations. Since HD maps are not updated in real-time, they face challenges with real-world changes such as physical damage or visual degradation like peeling paint. Furthermore, acquiring a dataset specialized exclusively for speed bumps through real-world driving is difficult.

We overcome these challenges through simulation. By receiving data directly from the simulator, we resolve the dependency on HD maps and can easily acquire vast amounts of specialized driving data for various types of speed bumps. Moreover, by leveraging the advantages of a simulation environment, we can include learning data from adverse weather conditions, where sensor performance degrades, and even extreme driving situations that are difficult to test due to safety concerns.

Additionally, we have incorporated V2V communication to broaden the scope of learning to handle a wider variety of on-road variables. Through this entire process, our system achieves optimal driving efficiency by ensuring safety while minimizing unnecessary deceleration.

System Requirement

Hardware Requirements

(The following specifications are the official recommendations for the smooth operation of the UC-win/Road simulator.)

- CPU: Intel Core i7 / AMD Ryzen 7 (10th Gen or newer)

- RAM: 32GB

- GPU: NVIDIA GeForce RTX 3060 or higher (VRAM 8GB+)

- Storage: 50GB+ free space (SSD recommended)

Software Requirements

- Operating System: Windows 10/11 (64-bit)

- Simulator: UC-win/Road Version 17.2.0 (32-bit)

- Language: Python 3.8.x (* A 32-bit version is required for API integration with UC-win/Road)

- External Python Libraries:

(* The following list includes only external libraries that need to be installed separately (e.g., via pip install). Python's built-in libraries are not listed.)

numpy: Used for core numerical computations, including matrix and vector operations.

Installation and Execution Guide

- Prerequisites

- Install UC-win/Road & Python:

Install UC-win/Road Version 17.2.0 and Python 3.8.x (32-bit). Please refer to the official UC-win/Road documentation to enable the Python API plugin.

(* The bit version of your Python installation must match the bit version of your UC-win/Road installation for the API to function correctly.)

- Download the Project Source Code: Clone the project repository using the following Git command

git clone https://github.com/5suk/CPWC.git

- Install Required Libraries: Run the following command to install the numpy library.

pip install numpy

- Initial Setup: Vehicle Braking Calibration

- Before running the main system, you must perform a one-time calibration to determine the vehicle's unique braking characteristics. This process generates a ‘speed_map.json’ file that is essential for calculating the optimal braking force.

- Launch UC-win/Road by opening the Autobump.rb project file. Once loaded, press the F10 key to hide the menu for a smoother experience.

- Set the initial driving speed within the UC-win/Road scenario to your desired test speed.

- Run the calibration script from your terminal.

python utils/calibaration.py

- When prompted in the terminal, enter the test speed. For more detailed and accurate data, it is recommended to test a range of speeds. A good approach is to test speeds from 55 km/h to 80 km/h in 5 km/h increments.

- The vehicle will automatically begin to decelerate when it detects a speed bump. When the vehicle collides with the bump, press the c key to signal the impact to the system. If you miss the timing or if the deceleration is incorrect, press b to discard the result for that run and try again.

- Repeat this process for all desired speeds. The system requires three successful impacts (three c presses) for each speed you test. The results are automatically saved and accumulated in the speed_map.json file.

- Run the Main System

- Launch UC-win/Road by opening the ‘Autobump.rb’ project file. Once loaded, press the F10 key to hide the menu for a smoother experience. (* You must verify that the and views are correctly displayed on the screen.)

- Execute the main.py script from the terminal.

python main.py

- Verifying Logs

You can monitor the system's real-time performance and review results by checking the log file located at ‘config/log.txt.’

System Architecture

Module Roles and Key Functions

This system is integrated with the simulator via its Python API and is composed of several functionally distinct Python modules. The overall execution flow is managed by main.py, which orchestrates each module to perform specific tasks and share results to achieve the system's objectives. To review the full source code, please visit https://github.com/5suk/CPWC.git.

- main.py (Main Controller): Manages the entire execution flow. It sets up and initializes a multiprocessing environment and communication queues for the independent operation of each module. It orchestrates all modules within the main loop to ensure the cohesive operation of the entire system.

- src/V2V.py (V2V Communication Module): Prepares for situations where a following vehicle's sensor view is obstructed due to close proximity to the leading vehicle. It simulates the role of transmitting geometry information, captured by a leading vehicle as it passes over a speed bump, to the following vehicle, thereby compensating for the limitations of sensor perception.

- src/vision.py (Perception & Classification Module): Based on the two views (front camera view, Dense Height Map) from the simulator, it determines the presence and geometry of a speed bump through pattern recognition and image analysis. Subsequently, it classifies the type of the speed bump according to this information.

- src/control.py (Control Module): Formulates an optimal deceleration plan by predicting ride comfort, which is calculated by combining various data points such as the speed bump's geometry and the vehicle's current speed. The vehicle's deceleration is then precisely managed using a calculated PWM (Pulse-Width Modulation) value.

- src/evaluate.py (Evaluation & Calibration Module): It evaluates the ride comfort prediction from the control module by comparing it against the actual ride comfort (GT-based) at the moment of impact. It also assesses the accuracy of the deceleration plan by analyzing the actual impact speed. This module performs self-learning by updating calibration coefficients based on these evaluation results to refine the process, utilizing an EMA (Exponential Moving Average) approach.

- utils/logger.py (Data Logging Module): Enables real-time monitoring of the entire process by recording data, and saves all final execution results into a txt file to support detailed post-analysis.

Key Variables List

| Category | Description | Variable Name (in code) | Abbreviation |
| --- | --- | --- | --- |
| Control |  |  |  |
| Predicted Ride Comfort | The anticipated ride comfort index before control begins. | prediction_RMS | pR |
| Ride Comfort Calibration | The calibration factor for ride comfort prediction, updated by the evaluate module. | pR_Calibration | pR_CAL |
| Braking Force | The final calculated braking force to be applied. | Brake_PWM | B_PWM |
| Braking Calibration | The calibration factor for braking force, updated by the evaluate module. | PWM_Calibration | PWM_CAL |
| Bump Distance | The remaining distance between the vehicle and the speed bump. | bump_distance | bD |
| Current Speed | The vehicle's current speed. | current_speed | cS |
| Target Speed | The desired speed to be reached after deceleration. | target_speed | tS |
| Pre-scanned Information | Pre-scanned speed bump information at the start of the scenario. | ThreeDModelGT_cache | 3DmodelGT_cache |
| Scanned Geometry Info | Geometry information of the model scanned via object data. | GT_Height, GT_Depth, GT_Distance | GT_H, GT_Dp, GT_Dt |
| Deceleration Status Flag | A status flag indicating whether deceleration is active. | Is_Controlling | Is_Controlling |
| Minimum Speed | The minimum achievable speed from the current speed with maximum braking. | minimum_Speed | mS |
| Predicted Comfort Level | The predicted ride comfort level felt by the user, based on the predicted RMS. | prediction_Level | pL |
| Perception |  |  |  |
| Measured Camera Values | Speed bump geometry information measured via the camera. | Measured_Height, Measured_Distance | M_H, M_Dt |
| Detection History | Stores consecutive detection results to improve reliability. | Detection_History | Detection_History |
| Bump Pattern | Indicates the presence of a speed bump pattern detected by the camera. | Bump_Pattern | Bump_Pattern |
| V2V Received Data | Data received via V2V communication. | v2v_data | v2v_data |
| Forward Vehicle Distance | Distance to the vehicle ahead, detected via V2V communication. | forward_vehicle_distance | vD |
| Evaluate |  |  |  |
| Actual Ride Comfort | The ride comfort index calculated upon actually passing the speed bump. | actual_RMS | aR |
| Ride Comfort Error | The error rate between predicted and actual ride comfort. | er_error | er_error |
| Speed Error | The error rate between the target speed and the actual passing speed. | ts_error | ts_error |

Code Details

- Inputs from Simulator

The system operates by receiving a variety of real-time data from the UC-win/Road simulator via its API. The primary inputs are as follows.

Real-time Views

Front Camera View: An RGB image data stream showing the road conditions ahead of the vehicle. It is primarily used for recognizing the paint patterns of speed bumps.

Dense Height Map: A view generated using an internal function of UC-win/Road (the Camera Shader Sample), which represents the height of an object from the ground level with color gradients. This can be considered the visualized result of fusing data from various sensors (like short-range LiDAR and a stereo camera) in a real vehicle.

Vehicle Information

Vehicle Information: Includes dynamic data such as the current speed of the primary vehicle. It also includes data from other vehicles in the simulation, which is utilized for calculating inter-vehicle distances for features like V2V communication.

Ground Truth Data

Speed Bump Geometry Information: The actual height, depth, and position of speed bumps set in the scenario. This ground truth data is used by the evaluate module to measure the system's ride comfort prediction accuracy and evaluate its braking design performance. Additionally, it is used in the V2V module to determine the distance to the bump for the preceding vehicle and to detect collision events.

Scenario Information

Loads scenario information set with desired conditions, such as weather, the presence of a forward vehicle, default driving speed, and vehicle type.

- main.py

Serves as the entry point for the entire system and acts as an orchestrator for all submodule processes. It handles the execution of and communication between each process.

- Setting Up the Logger

To record and output all real-time system operations, it configures the logging environment by calling a function from the utils/logger.py module.

# from main.py

setup_logging_area()

- Connecting to UC-win/Road and Setting Up the Scenario (Initialization)

It initiates the API connection with the simulator and acquires control by creating an instance of the UC-win/Road class.

initialize_simulation()

- Configuring Communication Channels

It sets up Queues or Shared Memory (Value) for safe data exchange between independent processes.

Queue vs. Shared Memory (Value) : A Queue is used for passing multiple data messages sequentially, whereas Shared Memory is used for sharing a single variable that multiple processes can read from and write to directly.

eval_to_control_queue = multiprocessing.Queue()		#Queue

forward_vehicle_distance = multiprocessing.Value('d', 999.9)	#Shared Memory

- Running Each Module's Process

It creates and starts the main function of each module as a separate process.

p_module = Process(target=’module name’, args=(...))

p_module.start()

- V2V.py (src/V2V.py)

Simulates Vehicle-to-Vehicle (V2V) communication to prepare for hazardous situations beyond the sensor's perception range, especially when the following vehicle's view is obstructed by a leading vehicle.

- Initializing the Speed Bump Object Dictionary

To prevent performance degradation from fetching data repeatedly during the drive, it initializes at the start of the scenario by receiving and storing speed bump information.

# from V2V.py

if any(instance_name_lower.startswith(target) for target in target_names):

height, depth = 0.0, 0.0

if instance.BoundingBoxesCount > 0:

bbox = instance.BoundingBox(0)

height = math.sqrt(bbox.yAxis.X**2 + bbox.yAxis.Y**2 + bbox.yAxis.Z**2) * 2

depth = math.sqrt(bbox.zAxis.X**2 + bbox.zAxis.Y**2 + bbox.zAxis.Z**2) * 2

ThreeDModelGT_cache.append({

"id": instance.ID, "name": instance.Name, "position": instance.Position,

"height": height, "width": depth

})

- Searching for Vehicles

It searches for the vehicle directly ahead that is within a certain proximity among all vehicles in the scenario.

front_car = my_car.DriverAheadInTraffic(const._primaryPath)

- Calculating Distance to the Forward Vehicle

It measures the distance to the forward vehicle to determine if it is within a range where the driver's view might be obstructed.

my_pos = my_car.Position

front_pos = front_car.Position

Vehicle_Distance.value = Distance(my_pos, front_pos)

- Checking Transmission Conditions and Sending Data

It determines that V2V data transmission is necessary if the forward vehicle has just passed a speed bump (a collision event occurs) and the distance to the following vehicle is within the communication range. It then sends the information of that speed bump.

if Vehicle_Distance.value <= 30.0:

# ...

if dist_front_to_bump < TRIGGER_DISTANCE_BUMP:

# …

v2v_to_vision_queue.put_nowait(data_packet)

- vision.py (src/vision.py)

Checks if the forward view is obstructed. If not, it analyzes visual information from the simulator to detect and classify speed bumps, serving as a trigger for the control module.

- Checking for Obstructed Forward View

It checks the distance to the forward vehicle (shared by the V2V module) to determine if the view is obstructed. If the distance is close, creating an obstructed view (Is_V2V is True), it bypasses its own analysis and instead relays the information received from the V2V module directly to the control module.

# from vision.py

if forward_vehicle_distance is not None:

Is_V2V = (forward_vehicle_distance.value <= 30.0)

if Is_V2V:

# Logic to pass v2v_data to the control module

- Detecting Speed Bumps Using Two Views

If the view is not obstructed, it comprehensively analyzes two views to detect speed bumps.

- Front Camera View

It analyzes the image to identify the presence of the yellow paint pattern of a speed bump. This process involves converting the input image to the HSV color space for better analysis, creating a mask to extract only yellow pixels using a defined color range, and then combining this mask with a predefined Region of Interest (ROI) mask to filter for yellow pixels exclusively within the road area. The system determines that a pattern exists only if the total number of these filtered pixels is greater than a set threshold. In short, it identifies the presence of the pattern by the number of yellow pixels within the ROI.

def detect_bump_pattern(frame, roi_mask, pattern_settings):

- Dense Height Map

This view simulates the output of a fused sensor system (e.g., short-range LiDAR and stereo camera) on a real vehicle, representing height from the ground with color (hue). The image is converted to the HSV color space, and pixels that are not in the blue hue range (ground) are masked. From this masked area, contours are extracted. The system analyzes the area and aspect ratio of these contours to determine if they match the geometry of a speed bump. Finally, it calculates the height (Measured_Height) from the average color of the confirmed contour area and the distance (Measured_Distance) from its vertical position within the ROI.

def analyze_bump_height_map(frame, roi_mask)

- Classifying Speed Bumps

It classifies the final type of the speed bump by synthesizing the pattern detection status and the speed bump's height information.

- Handling Depth Information

Once the type of the speed bump is confirmed, its depth information is also calculated and passed on. Advanced sensors (e.g., Solid-state LiDAR) can accurately measure an object's depth. In this simulation, we emulate the high-precision data such a sensor would provide by utilizing the Ground Truth (GT) for depth instead of measuring it from the height map.

GT_Depth_m = get_gt_depth(confirmed_type, ThreeDModelGT_cache, my_car)

- Ensuring Reliability and Transmitting Results

To handle temporary sensor failures or frame drops, the classified type is not transmitted immediately. The result is accumulated in Detection_History, and only when the same type is read consecutively over multiple frames is it deemed reliable and sent to the control module.

- control.py (src/control.py)

Synthesizes speed bump information received from the vision or V2V module with the vehicle's deceleration performance to formulate and execute an optimal deceleration plan based on ride comfort and efficiency.

- Applying Braking Capability Test Results

At startup, the module loads the vehicle's pre-measured braking capability test results (speed_map.json from utils/calibaration.py) to use as a baseline for all braking force calculations.

with open(config.CALIBRATION_DATA_FILE_PATH, 'r') as f:

speed_map_data = json.load(f)

- Formulating and Executing Deceleration Plan After Data Reception

Upon receiving speed bump information from vision, it formulates and executes a deceleration plan in the following sequence:

- Predict Ride Comfort: It calculates the predicted ride comfort (RMS) based on the bump's geometry and the maximum possible deceleration speed from the current speed.

minimum_Speed = estimate_min_speed_kmh(current_speed, speed_map_data)

prediction_RMS = calculate_rms(Measured_Height, depth_m, minimum_Speed / 3.6, pR_Calibration)

- Determine Target Speed: Based on the predicted RMS, it determines the optimal target speed that minimizes deceleration while maintaining a comfortable level.

prediction_Level = classify_rms(prediction_RMS)

# …

target_rms = config.COMFORT_TARGETS_RMS.get(prediction_Level, 0.5)

optimal_v_mps = solve_speed_for_target_rms(Measured_Height, depth_m, target_rms, pR_Calibration)

target_speed = max(minimum_Speed, (optimal_v_mps * 3.6 if optimal_v_mps else 0) + config.TARGET_SPEED_MARGIN_KMH)

- Calculate PWM: It calculates the optimal brake period (PWM) required to reach the target speed, considering the current speed and remaining distance.

Brake_PWM = calculate_brake_pwm(current_speed, target_speed, bump_distance, PWM_Calibration)

- Control Vehicle: It sends the calculated PWM value to main.py to apply actual control to the vehicle.

on_time = config.CONTROL_POLL_DT * Brake_PWM; off_time = config.CONTROL_POLL_DT * (1.0 - Brake_PWM)

car.Throttle = 0.0

if on_time > 0: car.ParkingBrake = True; time.sleep(on_time)

if off_time > 0: car.ParkingBrake = False; time.sleep(off_time)

- Transmitting Data for Deceleration Evaluation

At the moment the vehicle collides with the speed bump, it recalculates the actual ride comfort (RMS) using Ground Truth (GT) data and the actual impact speed. It then sends a comprehensive data packet—containing the predicted and actual RMS values, as well as the target and actual impact speeds—to the evaluate module to serve as the basis for the next control calibration.

- evaluat.py (src/evaluate.py)

Performing the self-learning function. It receives driving results from the control module, calculates the error between predicted and actual values, and calibrates control parameters to improve accuracy.

- Receiving Evaluation Data

It receives a data packet from the control module. This packet contains both the predicted values from the detection point (predicted rms, measured geometry) and the actual results from the passing point (actual passing speed, GT geometry).

- Calculating Actual Ride Comfort

Using the Ground Truth (GT) geometry of the speed bump and the actual passing speed received, it calculates the actual ride comfort (actual_RMS) experienced when the vehicle passed the bump.

# from evaluate.py

actual_RMS = calculate_actual_rms( evaluation_data['GT_Height'], evaluation_data['GT_Depth'], evaluation_data['actual_speed'])

- Calculating Errors

Based on the predictions and the newly calculated actual values, it calculates the ride comfort prediction error (er_error) and the speed control error (ts_error). These error values serve as the basis for determining how much to update the calibration coefficients.

er_error = (abs(prediction_RMS - actual_RMS)/prediction_RMS) * 100 if prediction_RMS > 0.01 else 0

ts_error = (abs(target_speed - current_speed) / target_speed) * 100 if target_speed > 0.01 else 0

- Updating Calibration

Based on the calculated errors, it progressively updates the ride comfort prediction coefficient (pR_Calibration) and the braking coefficient (PWM_Calibration) using an EMA (Exponential Moving Average) filter. This method allows for stable performance optimization as driving experience accumulates.

alpha_pr = config.LEARNING_RATE_PR_CALIBRATION

updated_pR_Calibration = (current_pR_Calibration * (1 - alpha_pr)) + (target_gain * alpha_pr)

alpha_pwm = config.LEARNING_RATE_PWM_CALIBRATION

updated_PWM_Calibration = (current_PWM_Calibration * (1 - alpha_pwm)) + (target_pwm_weight * alpha_pwm)

- Transmitting Updated Coefficients

It sends the newly calculated calibration back to the control module. The control module then immediately applies these values in its next deceleration plan, leading to more refined control with each subsequent run.